Title:  Gendrive - Description and Use              File: GENDRIVE.DOC

Date:   May 5, 1989

OVERVIEW

GENDRIVE is a utility that allows OrCAD users to modify the internals

of a generic driver so that a new driver can be created that provides

the proper functionality and performance for the graphics card.

GENDRIVE can handle most display adapters that claim to be EGA or VGA

compatible with resolutions up to 800 horizontal x 600 vertical.

CAUTION

A GENDRIVE produced driver has limited checking for adapter compatibility.

If video adapter and the new driver are not compatible, you may see either

a blank screen or a screen that has random looking patterns on it that do

not make sense. If you select one of the predefined driver configurations,

this shouldn't happen to you, but if it does, you may need to re-boot your

computer to get back to DOS by holding the Ctrl and Alt keys down and

pressing the Del key.

HOW TO USE

1) Switch to the directory containing GENDRIVE.EXE, GENDRIVE.DAT and

GENDRIVE.DRV.  This will normally be your DRIVER directory.  If you set

up your directory structure as OrCAD recommends, this will be:

\ORCAD\DRIVER

Type:  cd \orcad\driver        and press the ENTER key.

2) Invoke GENDRIVE.EXE

Type: gendrive                 and press the ENTER key.

3) GENDRIVE.EXE will initialize itself and read in the OrCAD supported

set of configurations from GENDRIVE.DAT.  It will then display the

driver description part of this as a set of menu choices.

At this point, there are 4 things you can do:

1) Type the number associated with your display adapter and press ENTER.

2) Press the M key to see More menu choices.

3) Press the Q key to Quit the GENDRIVE program. No updates will occur.

4) Press the S key to define a Special set of parameters.

If you type a number and press ENTER, the file GENDRIVE.DRV will be modified

with the associated set of parameters. You will be asked for a new file name

for the modified driver.  After getting the name, GENDRIVE.EXE writes the

modified driver to the disk and exits.

Title:  Gendrive - Description and Use                          Page 2

--------------------------------------------------------------------------

In most cases, you should be able to find a menu choice that supports

your display adapter. If you do, the rest of this document can be ignored.

--------------------------------------------------------------------------

If you press the S key, GENDRIVE.EXE will ask you to define the following

parameters:

1) The number of columns of pixels your display adapter supports.

Typical numbers here are 640, 752 and 800.

2) The number of rows of pixels your display adapter supports.

Typical numbers here are 350, 410, 480 and 600.

3) A value to be assigned to AX to set up for an Interrupt 10H.

4) A value to be assigned to BX to set up for an Interrupt 10H.

The AX and BX values must be consistent with the columns and rows

provided in 1 and 2 above.  The values for AX and BX can be gotten

from the technical reference or programmer's reference portion of

your display adapter's user manual.

Standard EGA/VGA values normally have AH=0 and AL equal either

10 (EGA 640x350) or 12 (VGA 640x480).   Hence, AX = 0010H or 0012H

for these.  Extended EGA cards often use AL=12H for 640x480 mode.

The values for sizes above 640x480 vary considerably.  In addition,

certain cards want to use a value for AH other than 0.

The GENDRIVE approach is designed to handle display adapters that follow

the general conventions of native mode EGA and VGA displays.

1) Display Buffer starts at address A000:0

2) There is less than 64K of addressable display memory.

Size of memory required by a display =  rows * cols / 8.

Example:  640x480 ==> 640 * 480 / 8 = 38400 bytes.

Note: 800x600 ==> 800 * 600 / 8 = 60000 bytes which is just less

than 64 kilobytes.  (1024x768 is too large.)

3) OrCAD uses 16 colors.  This implies that the Display Buffer

is really composed of 4 bit planes of 64K bytes each.

4) Pixels are stored 8 pixels per apparent byte. With the high order

bit of Byte A000:0 representing the upper left most bit on the screen.

5) The number of columns must be a multiple of 8. That is, no byte contains

bits from both the right end of a line and the left end of the next line.

(We know of no adapters that violate this rule.)

6) The mapping from bits on screen and in memory must be continuous.

For example, if byte n contains the last 8 bits on line m, then

byte (n+1) contains the first 8 bits on line (m+1).  (Various adapters

and modes violate this rule. For example, CGA and Hercules Graphics

modes.)

Title:  Gendrive - Description and Use                          Page 3

GENDRIVE Control File: GENDRIVE.DAT

GENDRIVE is controlled by an editable file called: GENDRIVE.DAT. You

may alter this file to make a Special (S) definition process into

menu choice.  You might do this if you have a large number of users

and a small variety of non-OrCAD-supported adapters.  If you choose

to edit the file, make sure your editor can output ASCII compatible

files containing only Carriage Return - Line Feed Sequences as line

separators.

GENDRIVE.DAT Format

1) Any line that starts with a semicolon is a comment.  Any line that does

not start with a comment must be a mode specification.

2) A mode specification consists of 5 fields separated by commas:

Driver Description, Columns, Rows, AX value, BX value.

- Driver Description: text that will display during application

initialization.  Normally contains Columns x Rows and display adapter

description.   This field can be enclosed in either single or double

quotes.  If the field contains an embedded comma, then it must be

enclosed in quotes.  If the field contains an embedded comma and a

single/double quote, use double/single quotes to enclose it.

Example Driver Descriptions:

EGA 640x350 16 color                       no quotes

"EGA 640x350 16 color"                     optional quotes

'EGA 640x350 16 color'                     optional quotes

"EGA 640x350, 16 color"                    required quotes (,)

"John's Favorite Display Card"             optional double quotes

John's Favorite Display Card               enclosing quote not needed

- Columns: the number of columns of pixels the display will support.

Enter this number in decimal. Example values are: 640, 752 and 800.

- Rows: the number of rows of pixels that the display will support.

Enter this number in decimal. Example values are: 350, 480 and 600.

- AX value: The value to be MOVed into AX before doing an INT 10H to

initialize the card. Enter this number in hexadecimal. (Hexadecimal

is used here because most manuals show the numbers in hex.)

Example values are: 0010H (EGA 640x350) and 0012H (VGA 640x480)

- BX value: The value to be MOVed into BX before doing an INT 10H to

initialize the card. Enter this number in hexadecimal. (Hexadecimal

is used here because most manuals show the numbers in hex.)

Example values are: 0H (most cards/modes) or 0010H or 0012H. The

latter two values cause 640x350 and 640x480 modes for some extended

EGA cards.